Parameter number by product discipline and parameter category

Product discipline 0 - Meteorological products, parameter category 15: radar

| CodeFlag | Description | Units | Note |
| --- | --- | --- | --- |
| 0 | Base spectrum width | m/s |  |
| 1 | Base reflectivity | dB |  |
| 2 | Base radial velocity | m/s |  |
| 3 | Vertically integrated liquid water (VIL) | kg m-2 |  |
| 4 | Layer-maximum base reflectivity | dB |  |
| 5 | Precipitation | kg m-2 | Note 2: Statistical process 1 (Accumulation) does not change units. It is recommended to use another parameter with "rate" in its name and accumulation in PDT. |
| 6 | Radar spectra (1) | - |  |
| 7 | Radar spectra (2) | - |  |
| 8 | Radar spectra (3) | - |  |
| 9 | Reflectivity of cloud droplets | dB |  |
| 10 | Reflectivity of cloud ice | dB |  |
| 11 | Reflectivity of snow | dB |  |
| 12 | Reflectivity of rain | dB |  |
| 13 | Reflectivity of graupel | dB |  |
| 14 | Reflectivity of hail | dB |  |
| 15 | Hybrid scan reflectivity | dB |  |
| 16 | Hybrid scan reflectivity height | m |  |
| 17 | Precipitation rate | kg m-2 s-1 |  |
| 18 | Radar data quality index | proportion | Note 175: Spatially analyzed quality indicator, according to OPERA II (Holleman I., Michelson D., Galli G., Germann U., and Peura M., 2006: Quality information for radars and radar data. OPERA II deliverable OPERA_2005_19), normalized to between 0 (poorest quality) to 1 (best quality). |
| 19 | Radar data quality flag | Code Table 4.106 |  |
| 20 | Layer-maximum precipitation rate | kg m-2 s-1 |  |
| 21-191 | Reserved |  |  |
| 192-254 | Reserved for local use |  |  |
| 255 | Missing |  |  |