Parameter number by product discipline and parameter category

Product discipline 0 - Meteorological products, parameter category 4: short-wave radiation

| CodeFlag | Description | Units | Note |
| --- | --- | --- | --- |
| 0 | Net short-wave radiation flux (surface) | W m-2 | Note 1: Parameter deprecated. See Regulation 92.6.2 and use another parameter instead. |
| 1 | Net short-wave radiation flux (top of atmosphere) | W m-2 | Note 1: Parameter deprecated. See Regulation 92.6.2 and use another parameter instead. |
| 2 | Short-wave radiation flux | W m-2 | Note 1: Parameter deprecated. See Regulation 92.6.2 and use another parameter instead. |
| 3 | Global radiation flux | W m-2 |  |
| 4 | Brightness temperature | K |  |
| 5 | Radiance (with respect to wave number) | W m-1 sr-1 |  |
| 6 | Radiance (with respect to wavelength) | W m-3 sr-1 |  |
| 7 | Downward short-wave radiation flux | W m-2 |  |
| 8 | Upward short-wave radiation flux | W m-2 |  |
| 9 | Net short wave radiation flux | W m-2 |  |
| 10 | Photosynthetically active radiation | W m-2 |  |
| 11 | Net short-wave radiation flux, clear sky | W m-2 |  |
| 12 | Downward UV radiation | W m-2 |  |
| 13 | Direct short-wave radiation flux | W m-2 |  |
| 14 | Diffuse short-wave radiation flux | W m-2 |  |
| 15 | Upward UV radiation emitted/reflected from the Earth's surface | W m-2 |  |
| 16-49 | Reserved |  |  |
| 50 | UV index (under clear sky) | Numeric | Note 24: The Global Solar UVI is formulated using the International Commission on Illumination (CIE) reference action spectrum for UV-induced erythema on the human skin (ISO17166:1999/CIE S 007/E-1998). It is a measure of the UV radiation that is relevant to and defined for a horizontal surface. The UVI is a unitless quantity defined by the formula: {{image}}. |
| 51 | UV index | Numeric | Note 24: The Global Solar UVI is formulated using the International Commission on Illumination (CIE) reference action spectrum for UV-induced erythema on the human skin (ISO17166:1999/CIE S 007/E-1998). It is a measure of the UV radiation that is relevant to and defined for a horizontal surface. The UVI is a unitless quantity defined by the formula: {{image}}. |
| 52 | Downward short-wave radiation flux, clear sky | W m-2 |  |
| 53 | Upward short-wave radiation flux, clear sky | W m-2 |  |
| 54 | Direct normal short-wave radiation flux | W m-2 | Note 69: Normal flux is on a surface lifted to be normal to sun rays. |
| 55 | UV visible albedo for diffuse radiation | % |  |
| 56 | UV visible albedo for direct radiation | % |  |
| 57 | UV visible albedo for direct radiation, geometric component | % |  |
| 58 | UV visible albedo for direct radiation, isotropic component | % |  |
| 59 | UV visible albedo for direct radiation, volumetric component | % |  |
| 60 | Photosynthetically active radiation flux, clear sky | W m-2 |  |
| 61 | Direct short-wave radiation flux, clear sky | W m-2 |  |
| 62 | Direct normal short-wave radiation flux, clear sky | W m-2 | Note 69: Normal flux is on a surface lifted to be normal to sun rays. |
| 63 | Diffuse short-wave radiation flux, clear sky | W m-2 |  |
| 64-191 | Reserved |  |  |
| 192-254 | Reserved for local use |  |  |
| 255 | Missing |  |  |