Parameter number by product discipline and parameter category

Product discipline 10 - Oceanographic products, parameter category 0: waves

| CodeFlag | Description | Units | Note |
| --- | --- | --- | --- |
| 0 | Wave spectra (1) | - |  |
| 1 | Wave spectra (2) | - |  |
| 2 | Wave spectra (3) | - |  |
| 3 | Significant height of combined wind waves and swell | m |  |
| 4 | Direction of wind waves | degree true |  |
| 5 | Significant height of wind waves | m |  |
| 6 | Mean period of wind waves | s |  |
| 7 | Direction of swell waves | degree true |  |
| 8 | Significant height of swell waves | m |  |
| 9 | Mean period of swell waves | s |  |
| 10 | Primary wave direction | degree true |  |
| 11 | Primary wave mean period | s |  |
| 12 | Secondary wave direction | degree true |  |
| 13 | Secondary wave mean period | s |  |
| 14 | Mean direction of combined wind waves and swell | degree true |  |
| 15 | Mean period of combined wind waves and swell | s |  |
| 16 | Coefficient of drag with waves | - |  |
| 17 | Friction velocity | m/s |  |
| 18 | Wave stress | N m-2 |  |
| 19 | Normalized wave stress | - |  |
| 20 | Mean square slope of waves | - |  |
| 21 | u-component surface Stokes drift | m/s |  |
| 22 | v-component surface Stokes drift | m/s |  |
| 23 | Period of maximum individual wave height | s |  |
| 24 | Maximum individual wave height | m |  |
| 25 | Inverse mean wave frequency | s |  |
| 26 | Inverse mean frequency of wind waves | s |  |
| 27 | Inverse mean frequency of total swell | s |  |
| 28 | Mean zero-crossing wave period | s |  |
| 29 | Mean zero-crossing period of wind waves | s |  |
| 30 | Mean zero-crossing period of total swell | s |  |
| 31 | Wave directional width | - |  |
| 32 | Directional width of wind waves | - |  |
| 33 | Directional width of total swell | - |  |
| 34 | Peak wave period | s |  |
| 35 | Peak period of wind waves | s |  |
| 36 | Peak period of total swell | s |  |
| 37 | Altimeter wave height | m |  |
| 38 | Altimeter corrected wave height | m |  |
| 39 | Altimeter range relative correction | - |  |
| 40 | 10-metre neutral wind speed over waves | m/s |  |
| 41 | 10-metre wind direction over waves | deg |  |
| 42 | Wave energy spectrum | m2 s rad-1 |  |
| 43 | Kurtosis of the sea-surface elevation due to waves | - |  |
| 44 | Benjamin-Feir index | - |  |
| 45 | Spectral peakedness factor | /s |  |
| 46 | Peak wave direction | deg |  |
| 47 | Significant wave height of first swell partition | m |  |
| 48 | Significant wave height of second swell partition | m |  |
| 49 | Significant wave height of third swell partition | m |  |
| 50 | Mean wave period of first swell partition | s |  |
| 51 | Mean wave period of second swell partition | s |  |
| 52 | Mean wave period of third swell partition | s |  |
| 53 | Mean wave direction of first swell partition | deg |  |
| 54 | Mean wave direction of second swell partition | deg |  |
| 55 | Mean wave direction of third swell partition | deg |  |
| 56 | Wave directional width of first swell partition | - |  |
| 57 | Wave directional width of second swell partition | - |  |
| 58 | Wave directional width of third swell partition | - |  |
| 59 | Wave frequency width of first swell partition | - |  |
| 60 | Wave frequency width of second swell partition | - |  |
| 61 | Wave frequency width of third swell partition | - |  |
| 62 | Wave frequency width | - |  |
| 63 | Frequency width of wind waves | - |  |
| 64 | Frequency width of total swell | - |  |
| 65 | Peak wave period of first swell partition | s |  |
| 66 | Peak wave period of second swell partition | s |  |
| 67 | Peak wave period of third swell partition | s |  |
| 68 | Peak wave direction of first swell partition | degree true |  |
| 69 | Peak wave direction of second swell partition | degree true |  |
| 70 | Peak wave direction of third swell partition | degree true |  |
| 71 | Peak direction of wind waves | degree true |  |
| 72 | Peak direction of total swell | degree true |  |
| 73 | Whitecap fraction | fraction |  |
| 74 | Mean direction of total swell | degree |  |
| 75 | Mean direction of wind waves | degree |  |
| 76 | Charnock | Numeric | Note 133: The Charnock parameter accounts for increased aerodynamic roughness as wave heights grow due to increasing surface stress. It depends on the wind speed, wave age and other aspects of the sea state and is used to calculate how much the waves slow down the wind. |
| 77 | Wave Spectral Skewness | Numeric |  |
| 78 | Wave energy flux magnitude | W m-1 |  |
| 79 | Wave energy flux mean direction | degree true |  |
| 80 | Ratio of wave angular and frequency width | Numeric |  |
| 81 | Free convective velocity over the oceans | m s-1 |  |
| 82 | Air density over the oceans | kg m-3 |  |
| 83 | Normalized energy flux into waves | Numeric | Note 134: Parameters are normalized by being divided by the product of air density and the square of the friction velocity. |
| 84 | Normalized stress into ocean | Numeric | Note 134: Parameters are normalized by being divided by the product of air density and the square of the friction velocity. |
| 85 | Normalized energy flux into ocean | Numeric | Note 134: Parameters are normalized by being divided by the product of air density and the square of the friction velocity. |
| 86 | Surface elevation variance due to waves (over all frequencies and directions) | m2 s rad-1 |  |
| 87 | Wave induced mean sea level correction | m |  |
| 88 | Spectral width index | Numeric |  |
| 89 | Number of events in freak waves statistics | Numeric |  |
| 90 | u-component of surface momentum flux into ocean | N m-2 |  |
| 91 | v-component of surface momentum flux into ocean | N m-2 |  |
| 92 | Wave turbulent energy flux into ocean | W m-2 |  |
| 93 | Envelop maximum individual wave height | m |  |
| 94 | Time domain maximum individual crest height | m |  |
| 95 | Time domain maximum individual wave height | m |  |
| 96 | Space time maximum individual crest height | m |  |
| 97 | Space time maximum individual wave height | m |  |
| 98 | Goda peakedness factor | Numeric |  |
| 99 | Benjamin-Feir index 2D (BFI2D) | Numeric | Note 179: BFI2D is calculated by dividing the BFI by an expression that increases with the directional spread and decreases with the peakedness. It comes from the consideration that an increase of the directional spread reduces the excess of kurtosis, which is not taken into account in the BFI alone. Mori, N., M. Onorato, and P. A. E. M. Janssen, 2011: On the Estimation of the Kurtosis in Directional Sea States for Freak Wave Forecasting. J. Phys. Oceanogr., 41, 1484–1497, https://doi.org/10.1175/2011JPO4542.1 |
| 100 | Crest-trough correlation | Numeric | Note 180: Crest-trough correlation is an estimate of the auto-correlation between the crest heights and the trough depths. In narrow-banded seas this parameter is quite high, because crests and troughs are approximately the same size. Gemmrich, J., Cicon, L. Generation mechanism and prediction of an observed extreme rogue wave. Sci Rep 12, 1718 (2022). https://doi.org/10.1038/s41598-022-05671-4 |
| 101 | X component of the wave radiative stress to sea-ice | N m-2 | Note 182: The x- and y- components of the wave radiative stress to sea-ice are not necessarily equivalent to the u- and v- components (eastward/northward). The x- and y- components strictly follow the defined coordinate system which may or may not follow the eastward and northward directions. |
| 102 | Y component of the wave radiative stress to sea-ice | N m-2 | Note 182: The x- and y- components of the wave radiative stress to sea-ice are not necessarily equivalent to the u- and v- components (eastward/northward). The x- and y- components strictly follow the defined coordinate system which may or may not follow the eastward and northward directions. |
| 103 | u-component of atmospheric surface momentum flux | N m-2 | Note 183: The atmospheric surface momentum flux is what the atmosphere is losing at the surface of the ocean. The surface momentum flux into the ocean is the momentum flux that is entering the ocean and driving the ocean circulation. The difference is that part of the atmospheric momentum flux goes into the waves and not into the ocean circulation or part of the momentum contained in the wave field that is transferred to the ocean when waves break. In short, the ocean surface has waves which modulate the momentum transfer between the atmosphere and the ocean interior. |
| 104 | v-component of atmospheric surface momentum flux | N m-2 | Note 183: The atmospheric surface momentum flux is what the atmosphere is losing at the surface of the ocean. The surface momentum flux into the ocean is the momentum flux that is entering the ocean and driving the ocean circulation. The difference is that part of the atmospheric momentum flux goes into the waves and not into the ocean circulation or part of the momentum contained in the wave field that is transferred to the ocean when waves break. In short, the ocean surface has waves which modulate the momentum transfer between the atmosphere and the ocean interior. |
| 105-191 | Reserved |  |  |
| 192-254 | Reserved for local use |  |  |
| 255 | Missing |  |  |