Parameter number by product discipline and parameter category

Product discipline 10 - Oceanographic products, parameter category 2: ice

| CodeFlag | Description | Units | Note |
| --- | --- | --- | --- |
| 0 | Ice cover | Proportion |  |
| 1 | Ice thickness | m |  |
| 2 | Direction of ice drift | degree true |  |
| 3 | Speed of ice drift | m/s |  |
| 4 | u-component of ice drift | m/s |  |
| 5 | v-component of ice drift | m/s |  |
| 6 | Ice growth rate | m/s |  |
| 7 | Ice divergence | /s |  |
| 8 | Ice temperature | K |  |
| 9 | Module of ice internal pressure | Pa m | Note 57: Ice internal pressure or stress (Pa m) is the integrated pressure across the vertical thickness of a layer of ice. It is produced when concentrated ice reacts to external forces such as wind and ocean currents. |
| 10 | Zonal vector component of vertically integrated ice internal pressure | Pa m |  |
| 11 | Meridional vector component of vertically integrated ice internal pressure | Pa m |  |
| 12 | Compressive ice strength | N/m |  |
| 13 | Snow temperature (over sea ice) | K |  |
| 14 | Albedo | Numeric |  |
| 15 | Sea ice volume per unit area | m3 m-2 |  |
| 16 | Snow volume over sea ice per unit area | m3 m-2 |  |
| 17 | Sea ice heat content | J m-2 |  |
| 18 | Snow over sea ice heat content | J m-2 |  |
| 19 | Ice freeboard thickness | m |  |
| 20 | Ice melt pond fraction | fraction |  |
| 21 | Ice melt pond depth | m |  |
| 22 | Ice melt pond volume per unit area | m3 m-2 |  |
| 23 | Sea ice fraction tendency due to parameterization | s-1 |  |
| 24 | x-component of ice drift | m s-1 |  |
| 25 | y-component of ice drift | m s-1 |  |
| 26 | Sea ice salinity | g kg-1 |  |
| 27 | Freezing/melting potential | W m-2 |  |
| 28 | Melt onset date | Numeric | Note 144: This is the Julian day of the year that surface melt begins. |
| 29 | Freeze onset date | Numeric | Note 145: This is the Julian day of the year that surface freeze begins. |
| 30 | Sea-ice breakup memory | Numeric | Note 181: This is a metric that quantifies the wave-induced mechanical fatigue on the sea-ice pack (0 means broken, 1 means solid). This metric only makes sense in the presence of sea ice. |
| 31 | Downward short-wave radiation flux | W m-2 |  |
| 32-191 | Reserved |  |  |
| 192-254 | Reserved for local use |  |  |
| 255 | Missing |  |  |