Fixed surface types and units

| CodeFlag | Description | Units | Note |
| --- | --- | --- | --- |
| 0 | Reserved |  |  |
| 1 | Ground or water surface | - |  |
| 2 | Cloud base level | - |  |
| 3 | Level of cloud tops | - |  |
| 4 | Level of 0 °C isotherm | - |  |
| 5 | Level of adiabatic condensation lifted from the surface | - |  |
| 6 | Maximum wind level | - |  |
| 7 | Tropopause | - |  |
| 8 | Nominal top of the atmosphere | - |  |
| 9 | Sea bottom | - |  |
| 10 | Entire atmosphere | - |  |
| 11 | Cumulonimbus (CB) base | m |  |
| 12 | Cumulonimbus (CB) top | m |  |
| 13 | Lowest level where vertically integrated cloud cover exceeds the specified percentage (cloud base for a given percentage cloud cover) | % |  |
| 14 | Level of free convection (LFC) | - |  |
| 15 | Convective condensation level (CCL) | - |  |
| 16 | Level of neutral buoyancy or equilibrium level (LNB) | - |  |
| 17 | Departure level of the most unstable parcel of air (MUDL) | - |  |
| 18 | Departure level of a mixed layer parcel of air with specified layer depth | Pa |  |
| 19 | Lowest level where cloud cover exceeds the specified percentage | % | Note 149: This level differs from entry 13 which is vertically accumulated from the surface while this entry is not. |
| 20 | Isothermal level | K |  |
| 21 | Lowest level where mass density exceeds the specified value (base for a given threshold of mass density) | kg m-3 |  |
| 22 | Highest level where mass density exceeds the specified value (top for a given threshold of mass density) | kg m-3 |  |
| 23 | Lowest level where air concentration exceeds the specified value (base for a given threshold of air concentration) | Bq m-3 |  |
| 24 | Highest level where air concentration exceeds the specified value (top for a given threshold of air concentration) | Bq m-3 |  |
| 25 | Highest level where radar reflectivity exceeds the specified value (echo top for a given threshold of reflectivity) | dBZ |  |
| 26 | Convective cloud layer base | m |  |
| 27 | Convective cloud layer top | m |  |
| 28-29 | Reserved |  |  |
| 30 | Specified radius from the centre of the Sun | m |  |
| 31 | Solar photosphere |  |  |
| 32 | Ionospheric D-region level |  |  |
| 33 | Ionospheric E-region level |  |  |
| 34 | Ionospheric F1-region level |  |  |
| 35 | Ionospheric F2-region level |  |  |
| 36 | Stratopause |  |  |
| 37-99 | Reserved |  |  |
| 100 | Isobaric surface | Pa |  |
| 101 | Mean sea level |  |  |
| 102 | Specific altitude above mean sea level | m |  |
| 103 | Specified height level above ground | m |  |
| 104 | Sigma level | "sigma" value |  |
| 105 | Hybrid level | - |  |
| 106 | Depth below land surface | m |  |
| 107 | Isentropic (theta) level | K |  |
| 108 | Level at specified pressure difference from ground to level | Pa |  |
| 109 | Potential vorticity surface | K m2 kg-1 s-1 |  |
| 110 | Reserved |  |  |
| 111 | Eta level | - | Note 85: The Eta vertical coordinate system involves normalizing the pressure at some point on a specific level by the mean sea-level pressure at that point. |
| 112 | Reserved |  |  |
| 113 | Logarithmic hybrid level |  |  |
| 114 | Snow level | Numeric |  |
| 115 | Sigma height level | - | Note 75: Sigma height level is the vertical model level of the height-based terrain-following coordinate (Gal-Chen and Somerville, 1975). The value of the level = (height of the level - height of the terrain) / (height of the top level - height of the terrain), which is greater than or equal to 0 and less than or equal to 1. |
| 116 | Reserved |  |  |
| 117 | Mixed layer depth | m |  |
| 118 | Hybrid height level | - | Note 54: Hybrid height level (Code figure 118) can be defined as:z(k) = A(k) + B(k) x orog(k = 1,...,NLevels; orog = orography; z(k) = height in metres at level k) |
| 119 | Hybrid pressure level | - | Note 55: Hybrid pressure level, for which Code figure 119 shall be used instead of 105, can be defined as:p(k) = A(k) + B(k) x sp(k = 1,...,NLevels; sp = surface pressure; p(k) = pressure at level k) |
| 120-149 | Reserved |  |  |
| 150 | Generalized vertical height coordinate |  | Note 84: The definition of a generalized vertical height coordinate implies the absence of coordinate values in Section 4 but the presence of an external 3D-GRIB message that specifies the height of every model grid point in metres (see Notes to Section 4 in the section above entitled Specification of Octet Contents), i.e., this GRIB message will contain the field with discipline = 0, category = 3, parameter = 6 (Geometric height). |
| 151 | Soil level | Numeric | Note 100: The soil level represents a model level for which the depth is not constant across the model domain. The depth in metres of the level is provided by another GRIB message with the parameter "soil depth" with discipline 2, category 3 and parameter number 27. |
| 152 | Sea-ice level | Numeric | Note 99: The sea-ice level represents a sea-ice model level for which the depth is not constant across the model domains. The depth in metres of the level is provided by another GRIB message with the parameter “sea-ice thickness” with discipline 10, category 2 and parameter number 1. |
| 153-159 | Reserved |  |  |
| 160 | Depth below sea level | m |  |
| 161 | Depth below water surface | m |  |
| 162 | Lake or river bottom | - |  |
| 163 | Bottom of sediment layer | - |  |
| 164 | Bottom of thermally active sediment layer | - |  |
| 165 | Bottom of sediment layer penetrated by thermal wave | - |  |
| 166 | Mixing layer | - |  |
| 167 | Bottom of root zone | - |  |
| 168 | Ocean model level | Numeric |  |
| 169 | Ocean level defined by water density (sigma-theta) difference from near-surface to level | kg m-3 | Note 18: The level is defined by a water property difference from the near-surface to the level. The near-surface is typically chosen at 10 m depth. The physical quantity used to compute the difference can be water density (σq) when using level type 169 or water potential temperature (θ) when using level type 170. |
| 170 | Ocean level defined by water potential temperature difference from near-surface to level | K | Note 18: The level is defined by a water property difference from the near-surface to the level. The near-surface is typically chosen at 10 m depth. The physical quantity used to compute the difference can be water density (σq) when using level type 169 or water potential temperature (θ) when using level type 170. |
| 171 | Ocean level defined by vertical eddy diffusivity difference from near-surface to level | m2 s-1 | Note 18: The level is defined by a water property difference from the near-surface to the level. The near-surface is typically chosen at 10 m depth. The physical quantity used to compute the difference can be water density (σq) when using level type 169 or water potential temperature (θ) when using level type 170. |
| 172 | Ocean level defined by water density (rho) difference from near-surface to level | m |  |
| 173 | Top of snow over sea ice on sea, lake or river | - |  |
| 174 | Top surface of ice on sea, lake or river | - |  |
| 175 | Top surface of ice, under snow cover, on sea, lake or river | - |  |
| 176 | Bottom surface (underside) ice on sea, lake or river | - |  |
| 177 | Deep soil (of indefinite depth) | - |  |
| 178 | Reserved | - |  |
| 179 | Top surface of glacier ice and inland ice | - |  |
| 180 | Deep inland or glacier ice (of indefinite depth) | - |  |
| 181 | Grid tile land fraction as a model surface | - |  |
| 182 | Grid tile water fraction as a model surface | - |  |
| 183 | Grid tile ice fraction on sea, lake or river as a model surface | - |  |
| 184 | Grid tile glacier ice and inland ice fraction as a model surface | - |  |
| 185 | Roof level | - |  |
| 186 | Wall level | - |  |
| 187 | Road level | - |  |
| 188 | Melt pond top surface | - |  |
| 189 | Melt pond bottom surface | - |  |
| 190 | Reserved |  |  |
| 191 | Abstract level with no vertical localization | - | Note 168: This level has no defined location along the vertical axis. Scale factor and scaled values of first and second fixed surface should be set to 'missing' if not used. |
| 192-254 | Reserved for local use |  |  |
| 255 | Missing |  |  |