cross-section grid with points equally spaced on the horizontal

| Octet No. | Count | Contents | Note | Code/Flag Table |
| --- | --- | --- | --- | --- |
| 15 | 1 | Shape of the Earth | (see Code table 3.2) | Code table 3.2 |
| 16 | 1 | Scale factor of radius of spherical Earth |  |  |
| 17-20 | 4 | Scaled value of radius of spherical Earth |  |  |
| 21 | 1 | Scale factor of major axis of oblate spheroid Earth |  |  |
| 22-25 | 4 | Scaled value of major axis of oblate spheroid Earth |  |  |
| 26 | 1 | Scale factor of minor axis of oblate spheroid Earth |  |  |
| 27-30 | 4 | Scaled value of minor axis of oblate spheroid Earth |  |  |
| 31-34 | 4 | Number of horizontal points |  |  |
| 35-38 | 4 | Basic angle of the initial production domain | Note 9: Basic angle of the initial production domain and subdivisions of this basic angle are provided to manage cases where the recommended unit of 10-6 degrees is not applicable to describe the longitudes and latitudes, and direction increments. For these descriptors, the unit is equal to the ratio of the basic angle and the subdivisions number. For ordinary cases, zero and missing values should be coded, equivalent to the respective values of 1 and 106 (10-6 degrees unit). |  |
| 39-42 | 4 | Subdivisions of basic angle used to define extreme longitudes and latitudes | Note 9: Basic angle of the initial production domain and subdivisions of this basic angle are provided to manage cases where the recommended unit of 10-6 degrees is not applicable to describe the longitudes and latitudes, and direction increments. For these descriptors, the unit is equal to the ratio of the basic angle and the subdivisions number. For ordinary cases, zero and missing values should be coded, equivalent to the respective values of 1 and 106 (10-6 degrees unit). |  |
| 43-46 | 4 | La1 - latitude of first grid point | Note 9: Basic angle of the initial production domain and subdivisions of this basic angle are provided to manage cases where the recommended unit of 10-6 degrees is not applicable to describe the longitudes and latitudes, and direction increments. For these descriptors, the unit is equal to the ratio of the basic angle and the subdivisions number. For ordinary cases, zero and missing values should be coded, equivalent to the respective values of 1 and 106 (10-6 degrees unit). |  |
| 47-50 | 4 | Lo1 - longitude of first grid point | Note 9: Basic angle of the initial production domain and subdivisions of this basic angle are provided to manage cases where the recommended unit of 10-6 degrees is not applicable to describe the longitudes and latitudes, and direction increments. For these descriptors, the unit is equal to the ratio of the basic angle and the subdivisions number. For ordinary cases, zero and missing values should be coded, equivalent to the respective values of 1 and 106 (10-6 degrees unit). |  |
| 51 | 1 | Scanning mode | (flags - see Flag table 3.4) | Flag table 3.4 |
| 52-55 | 4 | La2 - latitude of last grid point | Note 9: Basic angle of the initial production domain and subdivisions of this basic angle are provided to manage cases where the recommended unit of 10-6 degrees is not applicable to describe the longitudes and latitudes, and direction increments. For these descriptors, the unit is equal to the ratio of the basic angle and the subdivisions number. For ordinary cases, zero and missing values should be coded, equivalent to the respective values of 1 and 106 (10-6 degrees unit). |  |
| 56-59 | 4 | Lo2 - longitude of last grid point | Note 9: Basic angle of the initial production domain and subdivisions of this basic angle are provided to manage cases where the recommended unit of 10-6 degrees is not applicable to describe the longitudes and latitudes, and direction increments. For these descriptors, the unit is equal to the ratio of the basic angle and the subdivisions number. For ordinary cases, zero and missing values should be coded, equivalent to the respective values of 1 and 106 (10-6 degrees unit). |  |
| 60 | 1 | Type of horizontal line | (see Code table 3.20) | Code table 3.20 |
| 61-62 | 2 | Number of vertical points |  |  |
| 63 | 1 | Physical meaning of vertical coordinate | (see Code table 3.15) |  |
| 64 | 1 | Vertical dimension coordinate values definition | (see Code table 3.21) |  |
| 65-66 | 2 | NC - number of coefficients or values used to specify vertical coordinates |  |  |
| 67-(66+NCx4) |  | Coefficients to define vertical dimension coordinate values in functional form, or the explicit coordinate values (IEEE 32-bit floating-point values) |  |  |