triangular grid based on an icosahedron (see Part B, GRIB Attachment I)

| Octet No. | Count | Contents | Note | Code/Flag Table |
| --- | --- | --- | --- | --- |
| 15 | 1 | n2 - exponent of 2 for the number of intervals on main triangle sides |  |  |
| 16 | 1 | n3 - exponent of 3 for the number of intervals on main triangle sides |  |  |
| 17-18 | 2 | ni - number of intervals on main triangle sides of the icosahedron |  |  |
| 19 | 1 | nd - number of diamonds |  |  |
| 20-23 | 4 | Latitude of the pole point of the icosahedron on the sphere |  |  |
| 24-27 | 4 | Longitude of the pole point of the icosahedron on the sphere |  |  |
| 28-31 | 4 | Longitude of the centre line of the first diamond of the icosahedron on the sphere |  |  |
| 32 | 1 | Grid point position | (see Code table 3.8) | Code table 3.8 |
| 33 | 1 | Numbering order of diamonds | (see Flag table 3.9) | Flag table 3.9 |
| 34 | 1 | Scanning mode for one diamond | (see Flag table 3.10) | Flag table 3.10 |
| 35-38 | 4 | nt - total number of grid points |  |  |