Mercator

| Octet No. | Count | Contents | Note | Code/Flag Table |
| --- | --- | --- | --- | --- |
| 15 | 1 | Shape of the Earth | (see Code table 3.2) | Code table 3.2 |
| 16 | 1 | Scale factor of radius of spherical Earth |  |  |
| 17-20 | 4 | Scaled value of radius of spherical Earth |  |  |
| 21 | 1 | Scale factor of major axis of oblate spheroid Earth |  |  |
| 22-25 | 4 | Scaled value of major axis of oblate spheroid Earth |  |  |
| 26 | 1 | Scale factor of minor axis of oblate spheroid Earth |  |  |
| 27-30 | 4 | Scaled value of minor axis of oblate spheroid Earth |  |  |
| 31-34 | 4 | Ni - number of points along a parallel |  |  |
| 35-38 | 4 | Nj - number of points along a meridian |  |  |
| 39-42 | 4 | La1 - latitude of first grid point |  |  |
| 43-46 | 4 | Lo1 - longitude of first grid point |  |  |
| 47 | 1 | Resolution and component flags | (see Flag table 3.3) | Flag table 3.3 |
| 48-51 | 4 | LaD - latitude(s) at which the Mercator projection intersects the Earth (Latitude(s) where Di and Dj are specified) |  |  |
| 52-55 | 4 | La2 - latitude of last grid point |  |  |
| 56-59 | 4 | Lo2 - longitude of last grid point |  |  |
| 60 | 1 | Scanning mode | (see Flag table 3.4) | Flag table 3.4 |
| 61-64 | 4 | Orientation of the grid, angle between i direction on the map and the Equator | Note 49: Limited to the range of 0 to 90 degrees; if the angle of orientation of the grid is neither 0 nor 90 degrees, Di and Dj must be equal to each other. |  |
| 65-68 | 4 | Di - longitudinal direction grid length | Note 28: Grid length is in units of 10-3 m at the latitude specified by LaD. |  |
| 69-72 | 4 | Dj - latitudinal direction grid length | Note 28: Grid length is in units of 10-3 m at the latitude specified by LaD. |  |
| 73-nn |  | List of number of points along each meridian or parallel | Note 4: A quasi-regular grid is only defined for appropriate grid scanning modes. Either rows or columns, but not both simultaneously, may have variable numbers of points or variable spacing. The first point in each row (column) shall be positioned at the meridian (parallel) indicated by octets 47-54. The grid points shall be evenly spaced in latitude (longitude). ¦ Note 124: These octets are only present for quasi-regular grids. ¦ Note 136: Three parameters define a general latitude/longitude coordinate system, formed by a general rotation of the sphere. One choice for these parameters is: (a) The geographic latitude in degrees of the southern pole of the coordinate system, θp for example; (b) The geographic longitude in degrees of the southern pole of the coordinate system, λp for example; (c) The angle of rotation in degrees about the new polar axis (measured clockwise when looking from the southern to the northern pole) of the coordinate system, assuming the new axis to have been obtained by first rotating the sphere through λp degrees about the geographic polar axis, and then rotating through (90 + θp) degrees so that the southern pole moved along the (previously rotated) Greenwich meridian. [OR] See Note 2 under grid definition template 3.1 – rotated latitude/longitude (or equidistant cylindrical, or Plate Carrée). |  |