equatorial azimuthal equidistant projection

| Octet No. | Count | Contents | Note | Code/Flag Table |
| --- | --- | --- | --- | --- |
| 15 | 1 | Shape of the Earth | (see Code table 3.2) | Code table 3.2 |
| 16 | 1 | Scale factor of radius of spherical Earth |  |  |
| 17-20 | 4 | Scaled value of radius of spherical Earth |  |  |
| 21 | 1 | Scale factor of major axis of oblate spheroid Earth |  |  |
| 22-25 | 4 | Scaled value of major axis of oblate spheroid Earth |  |  |
| 26 | 1 | Scale factor of minor axis of oblate spheroid Earth |  |  |
| 27-30 | 4 | Scaled value of minor axis of oblate spheroid Earth |  |  |
| 31-34 | 4 | Nx - number of points along x-axis |  |  |
| 35-38 | 4 | Ny - number of points along y-axis |  |  |
| 39-42 | 4 | La1 - latitude of tangency point (centre of grid) |  |  |
| 43-46 | 4 | Lo1 - longitude of tangency point |  |  |
| 47 | 1 | Resolution and component flags | (see Flag table 3.3) | Flag table 3.3 |
| 48-51 | 4 | Dx - x-direction grid length in units of 10-3 m as measured at the point of the axis |  |  |
| 52-55 | 4 | Dy - y-direction grid length in units of 10-3 m as measured at the point of the axis |  |  |
| 56 | 1 | Projection centre flag |  |  |
| 57 | 1 | Scanning mode | (see Flag table 3.4) | Flag table 3.4 |