time section grid

| Octet No. | Count | Contents | Note | Code/Flag Table |
| --- | --- | --- | --- | --- |
| 15-18 | 4 | NT - number of time steps |  |  |
| 19 | 1 | Unit of offset from reference time | (see Code table 4.4) | Code table 4.4 |
| 20-23 | 4 | Offset from reference of first time (negative value when first bit set) |  |  |
| 24 | 1 | Type of time increment | (see Code table 4.11) | Code table 4.11 |
| 25 | 1 | Unit of time increment | (see Code table 4.4) | Code table 4.4 |
| 26-29 | 4 | Time increment (negative value when first bit set) |  |  |
|  |  | 30-36 Last date/time |  |  |
| 30-31 | 2 | Year |  |  |
| 32 | 1 | Month |  |  |
| 33 | 1 | Day |  |  |
| 34 | 1 | Hour |  |  |
| 35 | 1 | Minute |  |  |
| 36 | 1 | Second |  |  |
| 37-38 | 2 | Number of vertical points |  |  |
| 39 | 1 | Physical meaning of vertical coordinate | (see Code table 3.15) | Code table 3.15 |
| 40 | 1 | Vertical dimension coordinate values definition | (see Code table 3.21) | Code table 3.21 |
| 41-42 | 2 | NC - number of coefficients or values used to specify vertical coordinates |  |  |
| 43-(42+NCx4) |  | Coefficients to define vertical dimension coordinate values in functional form, or the explicit coordinate values (IEEE 32-bit floating-point values) |  |  |