azimuth-range projection

| Octet No. | Count | Contents | Note | Code/Flag Table |
| --- | --- | --- | --- | --- |
| 15-18 | 4 | Nb - number of data bins along radials | Note 1: A data bin is a data point representing the volume centred on it. |  |
| 19-22 | 4 | Nr - number of radials |  |  |
| 23-26 | 4 | La1 - latitude of centre point |  |  |
| 27-30 | 4 | Lo1 - longitude of centre point |  |  |
| 31-34 | 4 | Dx - spacing of bins along radials |  |  |
| 35-38 | 4 | Dstart - offset from origin to inner bound |  |  |
| 39 | 1 | Scanning mode | (see Flag table 3.4) | Flag table 3.4 |
|  |  | 40-(39+4Nr) For each of Nr radials |  |  |
| (40+4(X-1))-(41+4(X-1)) |  | Azi - starting azimuth, degrees x 10 (degrees as north) |  |  |
| (42+4(X-1))-(43+4(X-1)) |  | Adelta - azimuthal width, degrees x 100 (+ clockwise, - counterclockwise), with X = 1 to Nr |  |  |