transverse Mercator

| Octet No. | Count | Contents | Note | Code/Flag Table |
| --- | --- | --- | --- | --- |
| 15 | 1 | Shape of the Earth | (see Code table 3.2) | Code table 3.2 |
| 16 | 1 | Scale factor of radius of spherical Earth |  |  |
| 17-20 | 4 | Scaled value of radius of spherical Earth |  |  |
| 21 | 1 | Scale factor of major axis of oblate spheroid Earth |  |  |
| 22-25 | 4 | Scaled value of major axis of oblate spheroid Earth |  |  |
| 26 | 1 | Scale factor of minor axis of oblate spheroid Earth |  |  |
| 27-30 | 4 | Scaled value of minor axis of oblate spheroid Earth |  |  |
| 31-34 | 4 | Ni - number of points along i-axis |  |  |
| 35-38 | 4 | Nj - number of points along j-axis |  |  |
| 39-42 | 4 | LaR - geographic latitude of reference point |  |  |
| 43-46 | 4 | LoR - geographic longitude of reference point |  |  |
| 47 | 1 | Resolution and component flags | (see Flag table 3.3) | Flag table 3.3 |
| 48-51 | 4 | m - scale factor at reference point ratio of distance on map to distance on spheroid (IEEE 32-bit floating-point values) |  |  |
| 52-55 | 4 | XR - false easting, i-direction coordinate of reference point in units of 10-2 m |  |  |
| 56-59 | 4 | YR - false northing, j-direction coordinate of reference point in units of 10-2 m |  |  |
| 60 | 1 | Scanning mode | (see Flag table 3.4) | Flag table 3.4 |
| 61-64 | 4 | Di - i-direction increment length in units of 10-2 m |  |  |
| 65-68 | 4 | Dj - j-direction increment length in units of 10-2 m |  |  |
| 69-72 | 4 | x1 - i-direction coordinate of the first grid point in units of 10-2 m |  |  |
| 73-76 | 4 | y1 - j-direction coordinate of the first grid point in units of 10-2 m |  |  |
| 77-80 | 4 | x2 - i-direction coordinate of the last grid point in units of 10-2 m |  |  |
| 81-84 | 4 | y2 - j-direction coordinate of the last grid point in units of 10-2 m |  |  |