stretched and rotated latitude/longitude (or equidistant cylindrical, or Plate Carrée)

| Octet No. | Count | Contents | Note | Code/Flag Table |
| --- | --- | --- | --- | --- |
| 15-72 | 58 | Same as grid definition template 3.0 | Note 9: Basic angle of the initial production domain and subdivisions of this basic angle are provided to manage cases where the recommended unit of 10-6 degrees is not applicable to describe the longitudes and latitudes, and direction increments. For these descriptors, the unit is equal to the ratio of the basic angle and the subdivisions number. For ordinary cases, zero and missing values should be coded, equivalent to the respective values of 1 and 106 (10-6 degrees unit). |  |
| 73-76 | 4 | Latitude of the southern pole of projection |  |  |
| 77-80 | 4 | Longitude of the southern pole of projection |  |  |
| 81-84 | 4 | Angle of rotation of projection |  |  |
| 85-88 | 4 | Latitude of the pole of stretching |  |  |
| 89-92 | 4 | Longitude of the pole of stretching |  |  |
| 93-96 | 4 | Stretching factor |  |  |
| 97-nn |  | List of number of points along each meridian or parallel | Note 4: A quasi-regular grid is only defined for appropriate grid scanning modes. Either rows or columns, but not both simultaneously, may have variable numbers of points or variable spacing. The first point in each row (column) shall be positioned at the meridian (parallel) indicated by octets 47-54. The grid points shall be evenly spaced in latitude (longitude). ¦ Note 124: These octets are only present for quasi-regular grids. |  |