variable resolution latitude/longitude

| Octet No. | Count | Contents | Note | Code/Flag Table |
| --- | --- | --- | --- | --- |
| 15 | 1 | Shape of the Earth | (see Code table 3.2) | Code table 3.2 |
| 16 | 1 | Scale factor of radius of spherical Earth |  |  |
| 17-20 | 4 | Scaled value of radius of spherical Earth |  |  |
| 21 | 1 | Scale factor of major axis of oblate spheroid Earth |  |  |
| 22-25 | 4 | Scaled value of major axis of oblate spheroid Earth |  |  |
| 26 | 1 | Scale factor of minor axis of oblate spheroid Earth |  |  |
| 27-30 | 4 | Scaled value of minor axis of oblate spheroid Earth |  |  |
| 31-34 | 4 | Ni - number of points along a parallel |  |  |
| 35-38 | 4 | Nj - number of points along a meridian |  |  |
| 39-42 | 4 | Basic angle of the initial production domain | Note 9: Basic angle of the initial production domain and subdivisions of this basic angle are provided to manage cases where the recommended unit of 10-6 degrees is not applicable to describe the longitudes and latitudes, and direction increments. For these descriptors, the unit is equal to the ratio of the basic angle and the subdivisions number. For ordinary cases, zero and missing values should be coded, equivalent to the respective values of 1 and 106 (10-6 degrees unit). |  |
| 43-46 | 4 | Subdivisions of basic angle used to define extreme longitudes and latitudes, and direction increments | Note 9: Basic angle of the initial production domain and subdivisions of this basic angle are provided to manage cases where the recommended unit of 10-6 degrees is not applicable to describe the longitudes and latitudes, and direction increments. For these descriptors, the unit is equal to the ratio of the basic angle and the subdivisions number. For ordinary cases, zero and missing values should be coded, equivalent to the respective values of 1 and 106 (10-6 degrees unit). |  |
| 47 | 1 | Resolution and component flags | Note 114: The resolution flags (bits 3-4 of Flag table 3.3) are not applicable. | Flag table 3.3 |
| 48 | 1 | Scanning mode | (see Flag table 3.4) | Flag table 3.4 |
| 49-ii |  | List of longitudes | Note 9: Basic angle of the initial production domain and subdivisions of this basic angle are provided to manage cases where the recommended unit of 10-6 degrees is not applicable to describe the longitudes and latitudes, and direction increments. For these descriptors, the unit is equal to the ratio of the basic angle and the subdivisions number. For ordinary cases, zero and missing values should be coded, equivalent to the respective values of 1 and 106 (10-6 degrees unit). ¦ Note 85: The list of Ni longitudes and Nj latitudes shall be coded in the octets immediately following the grid definition template in octets 49 to ii and octets ii+1 to jj respectively, where ii = 48 + 4Ni and jj = 48 + 4Ni + 4Nj. |  |
| (ii+1)-jj |  | List of latitudes | Note 9: Basic angle of the initial production domain and subdivisions of this basic angle are provided to manage cases where the recommended unit of 10-6 degrees is not applicable to describe the longitudes and latitudes, and direction increments. For these descriptors, the unit is equal to the ratio of the basic angle and the subdivisions number. For ordinary cases, zero and missing values should be coded, equivalent to the respective values of 1 and 106 (10-6 degrees unit). ¦ Note 85: The list of Ni longitudes and Nj latitudes shall be coded in the octets immediately following the grid definition template in octets 49 to ii and octets ii+1 to jj respectively, where ii = 48 + 4Ni and jj = 48 + 4Ni + 4Nj. |  |