space view perspective or orthographic

| Octet No. | Count | Contents | Note | Code/Flag Table |
| --- | --- | --- | --- | --- |
| 15 | 1 | Shape of the Earth | (see Code table 3.2) | Code table 3.2 |
| 16 | 1 | Scale factor of radius of spherical Earth |  |  |
| 17-20 | 4 | Scaled value of radius of spherical Earth |  |  |
| 21 | 1 | Scale factor of major axis of oblate spheroid Earth |  |  |
| 22-25 | 4 | Scaled value of major axis of oblate spheroid Earth |  |  |
| 26 | 1 | Scale factor of minor axis of oblate spheroid Earth |  |  |
| 27-30 | 4 | Scaled value of minor axis of oblate spheroid Earth |  |  |
| 31-34 | 4 | Nx - number of points along x-axis (columns) |  |  |
| 35-38 | 4 | Ny - number of points along y-axis (rows or lines) |  |  |
| 39-42 | 4 | Lap - latitude of sub-satellite point |  |  |
| 43-46 | 4 | Lop - longitude of sub-satellite point |  |  |
| 47 | 1 | Resolution and component flags | (see Flag table 3.3) | Flag table 3.3 |
| 48-51 | 4 | dx - apparent diameter of Earth in grid lengths, in x-direction |  |  |
| 52-55 | 4 | dy - apparent diameter of Earth in grid lengths, in y-direction |  |  |
| 56-59 | 4 | Xp - x-coordinate of sub-satellite point (in units of 10-3 grid length expressed as an integer) |  |  |
| 60-63 | 4 | Yp - y-coordinate of sub-satellite point (in units of 10-3 grid length expressed as an integer) |  |  |
| 64 | 1 | Scanning mode | (see Flag table 3.4) | Flag table 3.4 |
| 65-68 | 4 | Orientation of the grid; i.e. the angle between the increasing y-axis and the meridian of the sub-satellite point in the direction of increasing latitude | Note 46: It is the angle between the increasing y-axis and the meridian 180°E if the sub-satellite point is the North Pole; or the meridian 0° if the sub-satellite point is the South Pole. |  |
| 69-72 | 4 | Nr - altitude of the camera from the Earth's centre, measured in units of the Earth's (equatorial) radius multiplied by a scale factor of 106 | Note 59: Octets 69–72 shall be set to all ones (missing) to indicate the orthographic view (from infinite distance). ¦ Note 22: For orthographic view from infinite distance, the value of Nr should be encoded as missing (all bits set to 1). ¦ Note 73: The apparent angular size of the Earth will be given by 2 x arcsin ((106)/Nr). |  |
| 73-76 | 4 | Xo - x-coordinate of origin of sector image |  |  |
| 77-80 | 4 | Yo - y-coordinate of origin of sector image |  |  |