cross-section of analysis and forecast, averaged or otherwise statistically processed over latitude or longitude

| Octet No. | Count | Contents | Note | Code/Flag Table |
| --- | --- | --- | --- | --- |
| 10 | 1 | Parameter category | (see Code table 4.1) | Code table 4.1 |
| 11 | 1 | Parameter number | (see Code table 4.2) | Code table 4.2 |
| 12 | 1 | Type of generating process | (see Code table 4.3) | Code table 4.3 |
| 13 | 1 | Background generating process identifier (defined by originating centre) |  |  |
| 14 | 1 | Analysis or forecast generating process identifier (defined by originating centre) |  |  |
| 15-16 | 2 | Hours of observational data cut-off after reference time | Note 33: Hours greater than 65534 will be coded as 65534. |  |
| 17 | 1 | Minutes of observational data cut-off after reference time |  |  |
| 18 | 1 | Indicator of unit of time range | (see Code table 4.4) | Code table 4.4 |
| 19-22 | 4 | Forecast time in units defined by octet 18 |  |  |
| 23 | 1 | Horizontal dimension processed | (see Code table 4.220) | Code table 4.220 |
| 24 | 1 | Treatment of missing data (e.g. below ground) | (see Code table 4.221) | Code table 4.221 |
| 25 | 1 | Type of statistical processing | (see Code table 4.10) | Code table 4.10 |
| 26-29 | 4 | Start of range |  |  |
| 30-33 | 4 | End of range |  |  |
| 34-35 | 2 | Number of values |  |  |