Individual ensemble forecast, control and perturbed, at a horizontal level or in a horizontal layer at a point in time for wave 2D spectra with frequencies and directions defined by formulae

| Octet No. | Count | Contents | Note | Code/Flag Table |
| --- | --- | --- | --- | --- |
| 10 | 1 | Parameter category | (see Code table 4.1) | Code table 4.1 |
| 11 | 1 | Parameter number | (see Code table 4.2) | Code table 4.2 |
| 12-13 | 2 | Wave direction number |  |  |
| 14-15 | 2 | Number of wave directions (ND) |  |  |
| 16-17 | 2 | Wave frequency number |  |  |
| 18-19 | 2 | Number of wave frequencies (NF) |  |  |
| 20 | 1 | Type of generating process | (see Code table 4.3) | Code table 4.3 |
| 21 | 1 | Background generating process identifier (defined by originating centre) |  |  |
| 22 | 1 | Analysis or forecast generating process identifier (defined by originating centre) |  |  |
| 23-24 | 2 | Hours of observational data cut-off after reference time | Note 33: Hours greater than 65534 will be coded as 65534. |  |
| 25 | 1 | Minutes of observational data cut-off after reference time |  |  |
| 26 | 1 | Indicator of unit of time range | (see Code table 4.4) | Code table 4.4 |
| 27-30 | 4 | Forecast time in units defined by previous octet |  |  |
| 31 | 1 | Type of ensemble forecast | (see Code table 4.6) | Code table 4.6 |
| 32 | 1 | Perturbation number |  |  |
| 33 | 1 | Number of forecasts in ensemble |  |  |
| 34 | 1 | Type of wave direction sequence | (see Code table 4.251) | Code table 4.251 |
| 35 | 1 | Number of wave direction sequence parameters (NDSP) |  |  |
|  |  | The next two entries are repeated NDSP times to encode the scale factors and scaled values of the sequence parameters. |  |  |
| 36+(NDSP-1)*5 |  | Scale factor of wave direction sequence parameter |  |  |
| (37+(NDSP-1)*5)-(40+(NDSP-1)*5) |  | Scaled value of wave direction sequence parameter | Note 162: The units of the wave directions are degree true. |  |
| 36+NDSP*5 |  | Type of wave frequency sequence | (see Code table 4.251) | Code table 4.251 |
| 37+NDSP*5 |  | Number of wave frequency sequence parameters (NFSP) |  |  |
|  |  | The next two entries are repeated NFSP times to encode the scale factors and scaled values of the sequence parameters. |  |  |
| 38+NDSP*5+(NFSP-1)*5 |  | Scale factor of wave frequency sequence parameter |  |  |
| (39+NDSP*5+(NFSP-1)*5)-(42+NDSP*5+(NFSP-1)*5) |  | Scaled value of wave frequency sequence parameter | Note 161: The frequencies are defined as 1/T where T is the wave period in seconds. |  |