Probability forecasts from large ensembles with spatiotemporal processing based on focal (moving window) statistics at a horizontal level or in a horizontal layer at a point in time

| Octet No. | Count | Contents | Note | Code/Flag Table |
| --- | --- | --- | --- | --- |
| 10 | 1 | Parameter category | (see Code table 4.1) | Code table 4.1 |
| 11 | 1 | Parameter number | (see Code table 4.2) | Code table 4.2 |
| 12 | 1 | Type of generating process | (see Code table 4.3) | Code table 4.3 |
| 13 | 1 | Background generating process identifier (defined by originating centre) |  |  |
| 14 | 1 | Forecast generating process identifier (defined by originating centre) |  |  |
| 15-16 | 2 | Hours after reference time of data cut-off | Note 33: Hours greater than 65534 will be coded as 65534. |  |
| 17 | 1 | Minutes after reference time of data cut-off |  |  |
| 18 | 1 | Indicator of unit of time range | (see Code table 4.4) | Code table 4.4 |
| 19-22 | 4 | Forecast time in units defined by octet 18 |  |  |
| 23 | 1 | Type of first fixed surface | (see Code table 4.5) | Code table 4.5 |
| 24 | 1 | Scale factor of first fixed surface |  |  |
| 25-28 | 4 | Scaled value of first fixed surface |  |  |
| 29 | 1 | Type of second fixed surface | (see Code table 4.5) | Code table 4.5 |
| 30 | 1 | Scale factor of second fixed surface |  |  |
| 31-34 | 4 | Scaled value of second fixed surface |  |  |
| 35 | 1 | Type of ensemble forecast | (see Code table 4.6) | Code table 4.6 |
| 36-39 | 4 | Number of forecasts in ensemble |  |  |
| 40 | 1 | Forecast probability number |  |  |
| 41 | 1 | Total number of forecast probabilities |  |  |
| 42 | 1 | Probability type | (see Code table 4.9) | Code table 4.9 |
| 43 | 1 | Scale factor of lower limit |  |  |
| 44-47 | 4 | Scaled value of lower limit |  |  |
| 48 | 1 | Scale factor of upper limit |  |  |
| 49-52 | 4 | Scaled value of upper limit |  |  |
| 53 | 1 | Spatial vicinity type | (see Code table 4.103) | Code table 4.103 |
| 54 | 1 | Number of spatial vicinity values (NSV) |  |  |
|  |  | The next entry repeats nsv=1:NSV times |  |  |
| (55+(nsv-1)*4)-(58+(nsv-1)*4) |  | Spatial vicinity value |  |  |
| 59+(nsv-1)*4 |  | Spatial vicinity processing | (see Code table 4.104) | Code table 4.104 |
| (60+(nsv-1)*4)-(61+(nsv-1)*4) |  | Spatial vicinity processing argument 1 (e.g., if previous is quantile) |  |  |
| (62+(nsv-1)*4)-(63+(nsv-1)*4) |  | Spatial vicinity processing argument 2 (e.g., if previous is quantile) |  |  |
| 64+(nsv-1)*4 |  | Spatial vicinity missing data | (see Code table 4.105) | Code table 4.105 |
| (65+(nsv-1)*4) |  | Temporal vicinity processing | (see Code table 4.104) | Code table 4.104 |
| (66+(nsv-1)*4) |  | Temporal vicinity unit | (see Code table 4.4) | Code table 4.4 |
| (67+(nsv-1)*4)-(70+(nsv-1)*4) |  | Temporal vicinity towards past |  |  |
| (71+(nsv-1)*4)-(74+(nsv-1)*4) |  | Temporal vicinity towards future |  |  |