Individual ensemble forecast, control and perturbed, at a horizontal level or in a horizontal layer at a point in time for radionuclides

| Octet No. | Count | Contents | Note | Code/Flag Table |
| --- | --- | --- | --- | --- |
| 10 | 1 | Parameter category | (see Code table 4.1) | Code table 4.1 |
| 11 | 1 | Parameter number | (see Code table 4.2) | Code table 4.2 |
| 12-13 | 2 | Atmospheric chemical constituent type | (see Code table 4.230) | Code table 4.230 |
| 14 | 1 | Source or sink | (see Code table 4.238) | Code table 4.238 |
| 15-16 | 2 | Transport model used | (See Code table 4.333) | Code table 4.333 |
| 17-18 | 2 | Requested by entity | (see Common Code table C-11) | Code table c-11 |
| 19-20 | 2 | Scenario origin | (See Code table 4.335) | Code table 4.335 |
| 21-22 | 2 | NWP used | (See Code table 4.336) | Code table 4.336 |
| 23-24 | 2 | Release start year |  |  |
| 25 | 1 | Release start month |  |  |
| 26 | 1 | Release start day |  |  |
| 27 | 1 | Release start hour |  |  |
| 28 | 1 | Release start minute |  |  |
| 29 | 1 | Release start second |  |  |
| 30-31 | 2 | Wall clock initial time of execution (year) |  |  |
| 32 | 1 | Wall clock initial time of execution (month) |  |  |
| 33 | 1 | Wall clock initial time of execution (day) |  |  |
| 34 | 1 | Wall clock initial time of execution (hour) |  |  |
| 35 | 1 | Wall clock initial time of execution (minute) |  |  |
| 36 | 1 | Wall clock initial time of execution (second) |  |  |
| 37 | 1 | Type of generating process | (see Code table 4.3) | Code table 4.3 |
| 38 | 1 | Background generating process identifier (defined by originating centre) |  |  |
| 39 | 1 | Forecast generating process identifier (defined by originating centre) |  |  |
| 40-41 | 2 | Hours after reference time of data cut-off | Note 33: Hours greater than 65534 will be coded as 65534. |  |
| 42 | 1 | Minutes after reference time of data cut-off |  |  |
| 43 | 1 | Indicator of unit of time range | (see Code table 4.4) | Code table 4.4 |
| 44-47 | 4 | Forecast time in units defined by octet 43 |  |  |
| 48 | 1 | Type of first fixed surface | (see Code table 4.5) | Code table 4.5 |
| 49 | 1 | Scale factor of first fixed surface |  |  |
| 50-53 | 4 | Scaled value of first fixed surface |  |  |
| 54 | 1 | Type of second fixed surface | (see Code table 4.5) | Code table 4.5 |
| 55 | 1 | Scale factor of second fixed surface |  |  |
| 56-59 | 4 | Scaled value of second fixed surface |  |  |
| 60 | 1 | Type of ensemble forecast | (see Code table 4.6) | Code table 4.6 |
| 61 | 1 | Perturbation number |  |  |
| 62 | 1 | Number of forecasts in ensemble |  |  |