Average, accumulation, or extreme values or other statistically processed values at a horizontal level or in a horizontal layer in a continuous or non-continuous time interval for radionuclides

| Octet No. | Count | Contents | Note | Code/Flag Table |
| --- | --- | --- | --- | --- |
| 10 | 1 | Parameter category (see Code table 4.1) |  | Code table 4.1 |
| 11 | 1 | Parameter number (see Code table 4.2) |  | Code table 4.2 |
| 12-13 | 2 | Atmospheric chemical constituent type (see Code table 4.230) |  | Code table 4.230 |
| 14 | 1 | Source or sink (see Code table 4.238) |  | Code table 4.238 |
| 15-16 | 2 | Transport model used | (See Code table 4.333) | Code table 4.333 |
| 17-18 | 2 | Requested by entity | (see Common Code table C-11) | Code table c-11 |
| 19-20 | 2 | Scenario origin | (See Code table 4.335) | Code table 4.335 |
| 21-22 | 2 | NWP used | (See Code table 4.336) | Code table 4.336 |
| 23-24 | 2 | Release start year |  |  |
| 25 | 1 | Release start month |  |  |
| 26 | 1 | Release start day |  |  |
| 27 | 1 | Release start hour |  |  |
| 28 | 1 | Release start minute |  |  |
| 29 | 1 | Release start second |  |  |
| 30-31 | 2 | Wall clock initial time of execution (year) |  |  |
| 32 | 1 | Wall clock initial time of execution (month) |  |  |
| 33 | 1 | Wall clock initial time of execution (day) |  |  |
| 34 | 1 | Wall clock initial time of execution (hour) |  |  |
| 35 | 1 | Wall clock initial time of execution (minute) |  |  |
| 36 | 1 | Wall clock initial time of execution (second) |  |  |
| 37 | 1 | Type of generating process (see Code table 4.3) |  | Code table 4.3 |
| 38 | 1 | Background generating process identifier (defined by originating centre) |  |  |
| 39 | 1 | Analysis or forecast generating process identifier (defined by originating centre) |  |  |
| 40-41 | 2 | Hours after reference time of data cut-off | Note 33: Hours greater than 65534 will be coded as 65534. |  |
| 42 | 1 | Minutes after reference time of data cut-off |  |  |
| 43 | 1 | Indicator of unit of time range (see Code table 4.4) |  | Code table 4.4 |
| 44-47 | 4 | Forecast time in units defined by octet 43 | Note 111: The reference time in section 1 and the forecast time together define the beginning of the overall time interval. |  |
| 48 | 1 | Type of first fixed surface (see Code table 4.5) |  | Code table 4.5 |
| 49 | 1 | Scale factor of first fixed surface |  |  |
| 50-53 | 4 | Scaled value of first fixed surface |  |  |
| 54 | 1 | Type of second fixed surface (see Code table 4.5) |  | Code table 4.5 |
| 55 | 1 | Scale factor of second fixed surface |  |  |
| 56-59 | 4 | Scaled value of second fixed surface |  |  |
| 60-61 | 2 | Year of end of overall time interval |  |  |
| 62 | 1 | Month of end of overall time interval |  |  |
| 63 | 1 | Day of end of overall time interval |  |  |
| 64 | 1 | Hour of end of overall time interval |  |  |
| 65 | 1 | Minute of end of overall time interval |  |  |
| 66 | 1 | Second of end of overall time interval |  |  |
| 67 | 1 | n - number of time range specifications describing the time intervals used to calculate the statistically processed field |  |  |
| 68-71 | 4 | Total number of data values missing in statistical process |  |  |
|  |  | 72-83 Specification of the outermost (or only) time range over which statistical processing is done |  |  |
| 72 | 1 | Statistical process used to calculate the processed field from the field at each time increment during the time range | (see Code table 4.10) | Code table 4.1 |
| 73 | 1 | Type of time increment between successive fields used in the statistical processing | (see Code table 4.11) | Code table 4.11 |
| 74 | 1 | Indicator of unit of time for time range over which statistical processing is done | (see Code table 4.4) | Code table 4.4 |
| 75-78 | 4 | Length of the time range over which statistical processing is done, in units defined by the previous octet |  |  |
| 79 | 1 | Indicator of unit of time for the increment between the successive fields used | (see Code table 4.4) | Code table 4.4 |
| 80-83 | 4 | Time increment between successive fields, in units defined by the previous octet |  |  |
|  |  | 84-nn These octets are included only if n > 1, where nn = 72 + 12 x n |  |  |
| 84-95 | 12 | As octets 72 to 83, next innermost step of processing |  |  |
| 96-nn |  | Additional time range specifications, included in accordance with the value of n. Contents as octets 72 to 83, repeated as necessary |  |  |