Verification scores for individual ensemble forecast, control and perturbed, at a horizontal level or in a horizontal layer at a point in time

| Octet No. | Count | Contents | Note | Code/Flag Table |
| --- | --- | --- | --- | --- |
| 10 | 1 | Parameter category | (see Code table 4.1) | Code table 4.1 |
| 11 | 1 | Parameter number | (see Code table 4.2) | Code table 4.2 |
| 12 | 1 | Type of generating process | (see Code table 4.3) | Code table 4.3 |
| 13 | 1 | Background process |  |  |
| 14 | 1 | Generating process identifier |  |  |
| 15-16 | 2 | Hours after data cut-off |  |  |
| 17 | 1 | Minutes after data cut-off |  |  |
| 18 | 1 | Indicator of unit of time range | (see Code table 4.4) | Code table 4.4 |
| 19-22 | 4 | Forecast time |  |  |
| 23 | 1 | Type of first fixed surface | (see Code table 4.5) | Code table 4.5 |
| 24 | 1 | Scale factor of first fixed surface |  |  |
| 25-28 | 4 | Scaled value of first fixed surface |  |  |
| 29 | 1 | Type of second fixed surface | (see Code table 4.5) | Code table 4.5 |
| 30 | 1 | Scale factor of second fixed surface |  |  |
| 31-34 | 4 | Scaled value of second fixed surface |  |  |
| 35 | 1 | Type of ensemble forecast | (see Code table 4.6) | Code table 4.6 |
| 36-39 | 4 | Perturbation number |  |  |
| 40-43 | 4 | Number of forecasts in ensemble |  |  |
| 44-45 | 2 | Verification score | (see Code table 4.120) | Code table 4.120 |
| 46 | 1 | Type of reference dataset for verification | (see Code table 4.121) | Code table 4.121 |
| 47 | 1 | Type of statistical processing over vertical for verification | (see Code table 4.10) | Code table 4.10 |
| 48 | 1 | Type of threshold operator for verification score | (see Code table 4.91) | Code table 4.91 |
| 49 | 1 | Type of additional arguments for verification score | (see Code table 4.122) | Code table 4.122 |
| 50 | 1 | Number of additional arguments for verification (NA) |  |  |
|  |  | The next two entries are repeated NA times na=0:NA – can be 0 times if no parameters are necessary (na=0) |  |  |
| 51 + (na-1)*5 | 1 | Scale factor of additional argument for verification |  |  |
| (52 + (na-1)*5) - (55 + (na-1)*5) | 4 | Scaled value of additional argument for verification |  |  |
|  |  | End of sequence repeated NA times |  |  |
| (51 + NA*5) - (52 + NA*5) | 2 | Year of start of verification period |  |  |
| 53 + NA*5 | 1 | Month of start of verification period |  |  |
| 54 + NA*5 | 1 | Day of start of verification period |  |  |
| 55 + NA*5 | 1 | Hour of start of verification period |  |  |
| 56 + NA*5 | 1 | Minute of start of verification period |  |  |
| 57 + NA*5 | 1 | Second of start of verification period |  |  |
| 58 + NA*5 | 1 | Number of verification period time ranges (NV) |  |  |
|  |  | The next five entries are repeated NV times nv=1:NV |  |  |
| 59 + NA*5 + (nv-1)*11 | 1 | Type of statistical processing for time range for verification period | (see Code table 4.10) | Code table 4.10 |
| 60 + NA*5 + (nv-1)*11 | 1 | Indicator of unit for time range for verification period | (see Code table 4.4) | Code table 4.4 |
| (61 + NA*5 + (nv-1)*11) - (64 + NA*5 + (nv-1)*11) | 4 | Length of time range for verification period |  |  |
| 65 + NA*5 + (nv-1)*11 | 1 | Indicator of unit for time increment for verification period | (see Code table 4.4) | Code table 4.4 |
| (66 + NA*5 + (nv-1)*11) - (69 + NA*5 + (nv-1)*11) | 4 | Time increment for verification period |  |  |
|  |  | End of sequence repeated NV times |  |  |
| (70 + NA*5 + NV*11) - (71 + NA*5 + NV*11) | 2 | Number of forecasts in verification |  |  |