individual ensemble forecast, control and perturbed, at a horizontal level or in a horizontal layer in a continuous or non-continuous time interval for atmospheric chemical constituents based on a distribution function

| Octet No. | Count | Contents | Note | Code/Flag Table |
| --- | --- | --- | --- | --- |
| 10 | 1 | Parameter category | (see Code table 4.1) | Code table 4.1 |
| 11 | 1 | Parameter number | (see Code table 4.2) | Code table 4.2 |
| 12-13 | 2 | Atmospheric chemical constituent type | (see Code table 4.230) | Code table 4.230 |
| 14-15 | 2 | Number of modes (N) of distribution | Note 36: If Number of modes (N) > 1, then between x N fields with mode number l = 1, …, N define the distribution function. x is the number of variable parameters in the distribution function. |  |
| 16-17 | 2 | Mode number (l) |  |  |
| 18-19 | 2 | Type of distribution function | Note 19: For more information, see Part B, GRIB Attachment III. | Code table 4.240 |
| 20 | 1 | Number of following function parameters (Np), defined by type given in octets 18-19 (Type of distribution function) |  |  |
|  |  | Repeat the following 5 octets for the number of function parameters (n = 1, Np), if Np > 0 |  |  |
| 21+5(n-1) |  | List of scale factor of fixed distribution function parameter (p1-pNp), defined by type of distribution in octets 18-19 |  |  |
| (22+5(n-1))-(25+5(n-1)) |  | List of scaled value of fixed distribution function parameter (p1-pNp), defined by type of distribution in octets 18-19 |  |  |
| 21+5Np |  | Type of generating process | (see Code table 4.3) | Code table 4.3 |
| 22+5Np |  | Background generating process identifier (defined by originating centre) |  |  |
| 23+5Np |  | Analysis or forecast generating process identifier (defined by originating centre) |  |  |
| (24+5Np)-(25+5Np) |  | Hours of observational data cut-off after reference time | Note 33: Hours greater than 65534 will be coded as 65534. |  |
| 26+5Np |  | Minutes of observational data cut-off after reference time |  |  |
| 27+5Np |  | Indicator of unit of time range | (see Code table 4.4) | Code table 4.4 |
| (28+5Np)-(31+5Np) |  | Forecast time in units defined by the previous octet | Note 111: The reference time in section 1 and the forecast time together define the beginning of the overall time interval. |  |
| 32+5Np |  | Type of first fixed surface | (see Code table 4.5) | Code table 4.5 |
| 33+5Np |  | Scale factor of first fixed surface |  |  |
| (34+5Np)-(37+5Np) |  | Scaled value of first fixed surface |  |  |
| 38+5Np |  | Type of second fixed surface | (see Code table 4.5) | Code table 4.5 |
| 39+5Np |  | Scale factor of second fixed surface |  |  |
| (40+5Np)-(43+5Np) |  | Scaled value of second fixed surface |  |  |
| 44+5Np |  | Type of ensemble forecast | (see Code table 4.6) | Code table 4.6 |
| 45+5Np |  | Perturbation number |  |  |
| 46+5Np |  | Number of forecasts in ensemble |  |  |
| (47+5Np)-(48+5Np) |  | Year - Time of end of overall time interval |  |  |
| (49+5Np) |  | Month - Time of end of overall time interval |  |  |
| (50+5Np) |  | Day - Time of end of overall time interval |  |  |
| (51+5Np) |  | Hour - Time of end of overall time interval |  |  |
| (52+5Np) |  | Minute - Time of end of overall time interval |  |  |
| (53+5Np) |  | Second - Time of end of overall time interval |  |  |
| (54+5Np) |  | n - number of time range specifications describing the time intervals used to calculate the statistically processed field |  |  |
| (55+5Np)-(58+5Np) |  | Total number of data values missing in statistical process |  |  |
|  |  | (59+5Np)-(70+5Np) Specification of the outermost (or only) time range over which statistical processing is done |  |  |
| (59+5Np) |  | Statistical process used to calculate the processed field from the field at each time increment during the time range | (see Code table 4.10) | Code table 4.10 |
| (60+5Np) |  | Type of time increment between successive fields used in the statistical processing | (see Code table 4.11) | Code table 4.11 |
| (61+5Np) |  | Indicator of unit of time for time range over which statistical processing is done | (see Code table 4.4) | Code table 4.4 |
| (62+5Np)-(65+5Np) |  | Length of the time range over which statistical processing is done, in units defined by the previous octet |  |  |
| (66+5Np) |  | Indicator of unit of time for the increment between the successive fields used | (see Code table 4.4) | Code table 4.4 |
| (67+5Np)-(70+5Np) |  | Time increment between successive fields, in units defined by the previous octet | Note 7: An increment of zero means that the statistical processing is the result of a continuous (or near-continuous) process, not the processing of a number of discrete samples. Examples of such continuous processes are the temperatures measured by analogue maximum and minimum thermometers or thermographs, and the rainfall measured by a rain gauge. ¦ Note 110: The reference and forecast times are successively set to their initial values plus or minus the increment, as defined by the type of time increment. For all but the innermost (last) time range, the next inner range is then processed using these references and forecast times as the initial reference and forecast times. |  |
|  |  | (71+5Np)-nn These octets are included only if n > 1, where nn = (58+5Np) + 12 x n |  |  |
| (71+5Np)-(82+5Np) |  | As octets (59+5Np) to (70+5Np), next innermost step of processing |  |  |
| (83+5Np)-nn |  | Additional time range specifications, included in accordance with the value of n. Contents as octets (59+5Np) to (70+5Np), repeated as necessary |  |  |