analysis or forecast at a horizontal level or in a horizontal layer at a local time

| Octet No. | Count | Contents | Note | Code/Flag Table |
| --- | --- | --- | --- | --- |
| 10 | 1 | Parameter category | (see Code table 4.1) | Code table 4.1 |
| 11 | 1 | Parameter number | (see Code table 4.2) | Code table 4.2 |
| 12 | 1 | Type of generating process | (see Code table 4.3) | Code table 4.3 |
| 13 | 1 | Background generating process identifier (defined by originating centre) |  |  |
| 14 | 1 | Analysis or forecast generating process identifier (defined by originating centre) |  |  |
| 15 | 1 | Type of first fixed surface | (see Code table 4.5) | Code table 4.5 |
| 16 | 1 | Scale factor of first fixed surface |  |  |
| 17-20 | 4 | Scaled value of first fixed surface |  |  |
| 21 | 1 | Type of second fixed surface | (see Code table 4.5) | Code table 4.5 |
| 22 | 1 | Scale factor of second fixed surface |  |  |
| 23-26 | 4 | Scaled value of second fixed surface |  |  |
| 27 | 1 | Method used to derive the data field values at the local time specified in section 1 | (see Code table 4.248) | Code table 4.248 |
| 28 | 1 | n - number of analyses or forecasts used to create the composite data field at the local time specified in section 1 (n >= 1) |  |  |
|  |  | Octets 29-46 Specification of the analysis or forecast used in the processing (n = 1) |  |  |
| 29-30 | 2 | Year of the analysis or forecast used in the processing |  |  |
| 31 | 1 | Month of the analysis or forecast used in the processing |  |  |
| 32 | 1 | Day of the analysis or forecast used in the processing |  |  |
| 33 | 1 | Hour of the analysis or forecast used in the processing |  |  |
| 34 | 1 | Minute of the analysis or forecast used in the processing |  |  |
| 35 | 1 | Second of the analysis or forecast used in the processing |  |  |
| 36 | 1 | Indicator of units of forecast time | Note 70: Set code to missing if analysis. | Code table 4.4 |
| 37-40 | 4 | Forecast time | Note 70: Set code to missing if analysis. |  |
| 41 | 1 | Number of time increments of the forecast used in the processing |  |  |
| 42 | 1 | Indicator of units of time for the time increments |  |  |
| 43-46 | 4 | Time increments between successive forecast times |  |  |
|  |  | Octets 47-nn are included only if n > 1 where nn = 28 + 18 x n |  |  |
| 47-nn |  | (n-1) repetitions of sequence of octets 29-46 describing the next analyses or forecasts used in the processing |  |  |