categorical forecasts at a horizontal level or in a horizontal layer in a continuous or non-continuous time interval

| Octet No. | Count | Contents | Note | Code/Flag Table |
| --- | --- | --- | --- | --- |
| 10 | 1 | Parameter category | (see Code table 4.1) | Code table 4.1 |
| 11 | 1 | Parameter number | (see Code table 4.2) | Code table 4.2 |
| 12 | 1 | Type of generating process | (see Code table 4.3) | Code table 4.3 |
| 13 | 1 | Background generating process identifier (defined by originating centre) |  |  |
| 14 | 1 | Forecast generating process identifier (defined by originating centre) |  |  |
| 15-16 | 2 | Hours after reference time of data cut-off | Note 33: Hours greater than 65534 will be coded as 65534. |  |
| 17 | 1 | Minutes after reference time of data cut-off |  |  |
| 18 | 1 | Indicator of unit of time range | (see Code table 4.4) | Code table 4.4 |
| 19-22 | 4 | Forecast time in units defined by octet 18 | Note 111: The reference time in section 1 and the forecast time together define the beginning of the overall time interval. |  |
| 23 | 1 | Type of first fixed surface | (see Code table 4.5) | Code table 4.5 |
| 24 | 1 | Scale factor of first fixed surface |  |  |
| 25-28 | 4 | Scaled value of first fixed surface |  |  |
| 29 | 1 | Type of second fixed surface | (see Code table 4.5) | Code table 4.5 |
| 30 | 1 | Scale factor of second fixed surface |  |  |
| 31-34 | 4 | Scaled value of second fixed surface |  |  |
| 35 | 1 | NC - number of categories |  |  |
|  |  | 36- Repeat the following 12 octets for each category (i = 1,NC) |  |  |
| (36+12(i-1)) |  | Code figure |  |  |
| (37+12(i-1)) |  | Type of interval for first and second limits | (see Code table 4.91) | Code table 4.91 |
| (38+12(i-1)) |  | Scale factor of first limit |  |  |
| (39+12(i-1))-(42+12(i-1)) |  | Scaled value of first limit |  |  |
| (43+12(i-1)) |  | Scale factor of second limit |  |  |
| (44+12(i-1))-(47+12(i-1)) |  | Scaled value of second limit |  |  |
| (48+12(NC-1))-(49+12(NC-1)) |  | Year of end of overall time interval |  |  |
| (50+12(NC-1)) |  | Month of end of overall time interval |  |  |
| (51+12(NC-1)) |  | Day of end of overall time interval |  |  |
| (52+12(NC-1)) |  | Hour of end of overall time interval |  |  |
| (53+12(NC-1)) |  | Minute of end of overall time interval |  |  |
| (54+12(NC-1)) |  | Second of end of overall time interval |  |  |
| (55+12(NC-1)) |  | n - number of time range specifications describing the time intervals used to calculate the statistically processed field |  |  |
| (56+12(NC-1))-(59+12(NC-1)) |  | Total number of data values missing in statistical process |  |  |
|  |  | 60-71 Specification of the outermost (or only) time range over which statistical processing is done |  |  |
| (60+12(NC-1)) |  | Statistical process used to calculate the processed field from the field at each time increment during the time range | (see Code table 4.10) | Code table 4.10 |
| (61+12(NC-1)) |  | Type of time increment between successive fields used in the statistical processing | (see Code table 4.11) | Code table 4.11 |
| (62+12(NC-1)) |  | Indicator of unit of time for time range over which statistical processing is done | (see Code table 4.4) | Code table 4.4 |
| (63+12(NC-1))-(66+12(NC-1)) |  | Length of the time range over which statistical processing is done, in units defined by the previous octet |  |  |
| (67+12(NC-1)) |  | Indicator of unit of time for the increment between the successive fields used | (see Code table 4.4) | Code table 4.4 |
| (68+12(NC-1))-(71+12(NC-1)) |  | Time increment between successive fields, in units defined by the previous octet | Note 7: An increment of zero means that the statistical processing is the result of a continuous (or near-continuous) process, not the processing of a number of discrete samples. Examples of such continuous processes are the temperatures measured by analogue maximum and minimum thermometers or thermographs, and the rainfall measured by a rain gauge. ¦ Note 157: The reference and forecast times are successively set to their initial values plus or minus the increment, as defined by the type of time increment (one of octets (60+12(NC-1)), (73+12(NC-1)), (85+12(NC-1)), ...). For all but the innermost (last) time range, the next inner range is then processed using these reference and forecast times as the initial reference and forecast times. |  |
|  |  | 72-nn These octets are included only if n > 1, where nn = 71+12(n-1)+12(NC-1) |  |  |
| (72+12(NC-1))-(83+12(NC-1)) |  | As octets (60+12(NC-1)) to (71+12(NC-1)), next innermost step of processing |  |  |
| (84+12(NC-1))-nn |  | Additional time range specifications, included in accordance with the value of n. Contents as octets (60+12(NC -1)) to (71+12(NC -1)), repeated as necessary |  |  |