Average, accumulation, extreme values or other statistically processed values of post-processing analysis or forecast at a horizontal level or in a horizontal layer at a local time

| Octet No. | Count | Contents | Note | Code/Flag Table |
| --- | --- | --- | --- | --- |
| 10 | 1 | Parameter category | (see Code table 4.1) | Code table 4.1 |
| 11 | 1 | Parameter number | (see Code table 4.2) | Code table 4.2 |
| 12-13 | 2 | Input process identifier | Note 82: The input process identifier shall have the value of the "analysis or forecast process identifier" of the original GRIB message used as input of the post-processing. |  |
| 14-15 | 2 | Input originating centre | Note 81: The input originating centre shall have the value of the "originating centre" of the original GRIB message used as input of the post-processing. | Code table CCT-11 |
| 16 | 1 | Type of post-processing | Note 126: This identifies which post-processing technique was used. This is defined by the originating centre. |  |
| 17 | 1 | Type of generating process | (see Code table 4.3) | Code table 4.3 |
| 18 | 1 | Background generating process identifier (defined by originating centre) |  |  |
| 19 | 1 | Analysis or forecast generating process identifier (defined by originating centre) |  |  |
| 20 | 1 | Type of first fixed surface | (see Code table 4.5) | Code table 4.5 |
| 21 | 1 | Scale factor of first fixed surface |  |  |
| 22-25 | 4 | Scaled value of first fixed surface |  |  |
| 26 | 1 | Type of second fixed surface | (see Code table 4.5) | Code table 4.5 |
| 27 | 1 | Scale factor of second fixed surface |  |  |
| 28-31 | 4 | Scaled value of second fixed surface |  |  |
| 32 | 1 | Statistical process used to calculate the fields that will be used in the local time processing | (see Code table 4.10) | Code table 4.10 |
| 33 | 1 | Indicator of unit of time range over which statistical processing is done | (see Code table 4.4) | Code table 4.4 |
| 34-37 | 4 | Length of the time range over which statistical processing is done in units defined by the previous octet | Note 140: This represents the length of time over which the statistical processing was applied. The local time defined in section 1 represents the end of this processing. For instance, a value of 24 h corresponds to a statistical processing between the previous day at local time and this day at local time. |  |
| 38 | 1 | Number of statistically processed fields used in the local time composite field | Note 141: This represents the number of statistically processed fields (or stripes) used to create the composite local time field. For instance, a value of 8 means that 8 statistically processed fields have been used in the processing, each of them representing a section of 45 degrees of longitude (360/8) centred around the UTC time corresponding to the local time. |  |
| 39 | 1 | Method used to derive the data field values at the local time specified in section 1 | (see Code table 4.248) | Code table 4.248 |
| 40 | 1 | n - number of analyses or forecasts used to create the composite data field at the local time specified in section 1 (n >= 1) | Note 146: This is the number of forecasts and time steps used to create the statistically processed fields. These implicitly have the same statistical process as defined in octet 32. If a forecast has 2 time increments (3 hourly day 1 to 5 then 6 hourly), it should be encoded as 2 forecasts with the same reference time, using the appropriate starting forecast time and time increments. |  |
|  |  | Octets 41-58 Specification of the analysis or forecast used in the processing (n = 1) |  |  |
| 41-42 | 2 | Year of the analysis or forecast used in the processing |  |  |
| 43 | 1 | Month of the analysis or forecast used in the processing |  |  |
| 44 | 1 | Day of the analysis or forecast used in the processing |  |  |
| 45 | 1 | Hour of the analysis or forecast used in the processing |  |  |
| 46 | 1 | Minute of the analysis or forecast used in the processing |  |  |
| 47 | 1 | Second of the analysis or forecast used in the processing |  |  |
| 48 | 1 | Indicator of units of forecast time (set to missing if analysis) | (see Code table 4.4) | Code table 4.4 |
| 49-52 | 4 | Forecast time | Note 151: The reference time in octets 41-47 and the forecast time together define the beginning of the overall time interval. ¦ Note 70: Set code to missing if analysis. |  |
| 53 | 1 | Number of time increments of the forecast used in the processing |  |  |
| 54 | 1 | Indicator of units of time for the time increments | (see Code table 4.4) | Code table 4.4 |
| 55-58 | 4 | Time increments between successive forecast times | Note 147: This also represents the length of time range of the statistically processed fields. For instance, to create a 24 h accumulation (encoded in octets 34-37), we could use several 3 h accumulations, or 6 h accumulations, a mixture of the two, etc. |  |
|  |  | Octets 59-nn are included only if n > 1 where nn = 40 + 18 x n |  |  |
| 59-nn |  | (n-1) repetitions of sequence of octets 41-58 describing the next analyses or forecasts used in the processing |  |  |