Matrix values at grid point - simple packing

| Octet No. | Count | Contents | Note | Code/Flag Table |
| --- | --- | --- | --- | --- |
| 12-21 | 10 | Same as data representation template 5.0 |  |  |
| 22 | 1 | 0, no matrix bit maps present; 1-matrix bit maps present |  |  |
| 23-26 | 4 | Number of data values encoded in Section 7 |  |  |
| 27-28 | 2 | NR - first dimension (rows) of each matrix |  |  |
| 29-30 | 2 | NC - second dimension (columns) of each matrix |  |  |
| 31 | 1 | First dimension coordinate value definition | (Code table 5.2) | Code table 5.2 |
| 32 | 1 | NC1 - number of coefficients or values used to specify first dimension coordinate function |  |  |
| 33 | 1 | Second dimension coordinate value definition | (Code table 5.2) | Code table 5.2 |
| 34 | 1 | NC2 - number of coefficients or values used to specify second dimension coordinate function |  |  |
| 35 | 1 | First dimension physical significance | (Code table 5.3) | Code table 5.3 |
| 36 | 1 | Second dimension physical significance | (Code table 5.3) | Code table 5.3 |
| 37-(36+NC1x4) |  | Coefficients to define first dimension coordinate values in functional form, or the explicit coordinate values (IEEE 32-bit floating-point value) |  |  |
| (37+NC1x4)-(36+4(NC1+NC2)) |  | Coefficients to define second dimension coordinate values in functional form, or the explicit coordinate values (IEEE 32-bit floating-point value) |  |  |