Grid point data - JPEG 2000 code stream format

| Octet No. | Count | Contents | Note | Code/Flag Table |
| --- | --- | --- | --- | --- |
| 12-15 | 4 | Reference value (R) (IEEE 32-bit floating-point value) |  |  |
| 16-17 | 2 | Binary scale factor (E) |  |  |
| 18-19 | 2 | Decimal scale factor (D) |  |  |
| 20 | 1 | Number of bits required to hold the resulting scaled and referenced data values (i.e. depth of the greyscale image) | Note 84: The JPEG 2000 standard specifies that the bit-depth must be in the range of 1 to 38 bits. |  |
| 21 | 1 | Type of original field values | (see Code table 5.1) |  |
| 22 | 1 | Type of compression used | (see Code table 5.40) |  |
| 23 | 1 | Target compression ratio, M:1 (with respect to the bit-depth specified in octet 20), when octet 22 indicates lossy compression. Otherwise, set to missing | Note 74: The compression ratio M:1 (e.g. 20:1) specifies that the encoded stream should be less than ((1/M) x depth x number of data points) bits, where depth is specified in octet 20 and the number of data points in octets 6–9 of the data representation section. |  |