Spherical harmonics data - complex packing

| Octet No. | Count | Contents | Note | Code/Flag Table |
| --- | --- | --- | --- | --- |
| 12-20 | 9 | Same as data representation template 5.50 |  |  |
| 21-24 | 4 | P - Laplacian scaling factor (expressed in 10-6 units) |  |  |
| 25-26 | 2 | JS - pentagonal resolution parameter of the unpacked subset | Note 119: The unpacked subset is a set of values defined in the same way as the full set of values (on a spectrum limited to JS, KS and MS), but on which scaling and packing are not applied. Associated values are stored in octets 6 onwards of Section 7. |  |
| 27-28 | 2 | KS - pentagonal resolution parameter of the unpacked subset | Note 119: The unpacked subset is a set of values defined in the same way as the full set of values (on a spectrum limited to JS, KS and MS), but on which scaling and packing are not applied. Associated values are stored in octets 6 onwards of Section 7. |  |
| 29-30 | 2 | MS - pentagonal resolution parameter of the unpacked subset | Note 119: The unpacked subset is a set of values defined in the same way as the full set of values (on a spectrum limited to JS, KS and MS), but on which scaling and packing are not applied. Associated values are stored in octets 6 onwards of Section 7. |  |
| 31-34 | 4 | TS - total number of values in the unpacked subset | Note 119: The unpacked subset is a set of values defined in the same way as the full set of values (on a spectrum limited to JS, KS and MS), but on which scaling and packing are not applied. Associated values are stored in octets 6 onwards of Section 7. |  |
| 35 | 1 | Precision of the unpacked subset | (see Code table 5.7) | Code table 5.7 |