spectral data for limited area models - complex packing

| Octet No. | Count | Contents | Note | Code/Flag Table |
| --- | --- | --- | --- | --- |
| 12-15 | 4 | Reference value (R) (IEEE 32-bit floating-point value) |  |  |
| 16-17 | 2 | Binary scale factor (E) |  |  |
| 18-19 | 2 | Decimal scale factor (D) |  |  |
| 20 | 1 | Number of bits used for each packed value (field width) |  |  |
| 21 | 1 | Bi-Fourier sub-truncation type | (see Code table 5.25) | Code table 5.25 |
| 22 | 1 | Packing mode for axes | (see Code table 5.26) | Code table 5.26 |
| 23-26 | 4 | P - Laplacian scaling factor (expressed in 10-6 units) |  |  |
| 27-28 | 2 | NS - bi-Fourier resolution parameter of the unpacked subset | Note 120: The unpacked subset is a set of values defined in the same way as the full set of values (on a spectrum limited to NS and MS), but on which scaling and packing are not applied. Associated values are stored in octets 6 onwards of Section 7. |  |
| 29-30 | 2 | MS - bi-Fourier resolution parameter of the unpacked subset | Note 120: The unpacked subset is a set of values defined in the same way as the full set of values (on a spectrum limited to NS and MS), but on which scaling and packing are not applied. Associated values are stored in octets 6 onwards of Section 7. |  |
| 31-34 | 4 | TS - total number of values in the unpacked subset | Note 120: The unpacked subset is a set of values defined in the same way as the full set of values (on a spectrum limited to NS and MS), but on which scaling and packing are not applied. Associated values are stored in octets 6 onwards of Section 7. |  |
| 35 | 1 | Precision of the unpacked subset | (see Code table 5.7) | Code table 5.7 |